Dataset

Dataset Overview:

- Fault Types:

  - Normal Condition: Sensor data under normal operating conditions.

  - Fault-A to Fault-D: Sensor data under different fault scenarios with varying loads (2 kg to 10 kg).

- Data Specifications:

  - Sampling Rate: 1600 Hz (simulated).

  - Total Samples: 57,299, with 17 columns.

  - Analog Signals: Columns 0 to 10 (e.g., currents, accelerometer axes, sound sensors).

  - Digital Signals: Columns 11 to 16 (e.g., water level sensors, tachometers, relays).

- Dataset Breakdown:

  - Normal Condition: 15,158 samples.

  - Fault-A (2 kg): 10,574 samples.

  - Fault-B (5 kg): 10,514 samples.

  - Fault-C (7 kg): 10,526 samples.

  - Fault-D (10 kg): 10,527 samples.

Column Descriptions:

- Columns 0–2 (Currents): Measure current in three phases (A, B, C).

- Columns 3–5 (Accelerometer1): Vibration data (X, Y, Z axes) before the gearbox.

- Columns 6–8 (Accelerometer2): Vibration data (X, Y, Z axes) after the gearbox.

- Columns 9–10 (Sound Sensors): Acoustic data before and after the gearbox.

- Columns 11–12 (Water Level Sensors): Digital inputs for monitoring water levels.

- Columns 13–14 (Tachometers): Digital inputs for rotational speed.

- Columns 15–16 (Relays): Digital outputs for controlling system components.

Potential Use Cases:

- Fault Diagnosis and Classification:

  - Identify the type of fault (A, B, C, or D) based on sensor data.

- Prognostics:

  - Predict the progression of faults and potential failures.

- System Monitoring:

  - Real-time detection of abnormal conditions using accelerometer and sound sensor data.

- Correlation Analysis:

  - Analyze the relationship between various sensor readings and fault conditions.

- AI Model Development:

  - Train machine learning or deep learning models for fault detection and prediction.